## APPENDICES

### Appendix A: 	Map showing locations which were assed in the techno-economic model, with annual global average solar radiation (Gav), and annual dry bulb temperature range (Tdb) (ºC) (annual average between brackets) per location.

### Appendix B: Major equipment specifications and costs

Table B.1 - Major Equipment Capacities and Costs (corrected for inflation)

| Details | Capacity |  | Costs·unit-1 (€) | Power Req. (kWh) | Efficiency |
| --- | --- | --- | --- | --- | --- |
| Pumps |  |  |  |  |  |
| Pump (seawater) | 2 | m3·h-1 | € 480 | 0.18 |  |
|  | 4 | m3·h-1 | € 1,091 | 0.4 |  |
|  | 20 | m3·h-1 | € 3,163 | 12 |  |
|  | 200 | m3·h-1 | € 14,292 | 5.9 |  |
| Culture circulation pump | 700 | m3·h-1 | € 29,628 |  | 0.75 |
|  | 7000 | m3·h-1 | € 206,623 |  | 0.75 |
|  | 28000 | m3·h-1 | € 628,510 |  | 0.75 |
| Tanks |  |  |  |  |  |
| Liquid Holding Tank | 10 | m3 | € 2,556 |  |  |
|  | 20 | m3 | € 4,711 |  |  |
|  | 50 | m3 | € 7,337 |  |  |
|  | 400 | m3 | € 48,493 |  |  |
|  | 4000 | m3 | € 501,539 |  |  |
| Sterilization Process |  |  |  |  |  |
| Medium Filter Unit | 5.99 | m3·h-1 | € 16,010 |  |  |
|  | 59.9 | m3·h-1 | € 113,343 |  |  |
| Mixing Unit |  |  |  |  |  |
| Media Mixing Unit | 0.1 | m3 | € 14,759 | 0.05184 |  |
|  | 4 | m3 | € 231,924 | 2.0736 |  |
|  | 8 | m3 | € 256,171 | 4.1472 |  |
|  | 25 | m3 | € 306,772 | 12.96 |  |
| CO2 Supply |  |  |  |  |  |
| CO2 Supply Unit | 1 | ha-1 | € 4,978 |  |  |
| Aeration (for degasser and flat panel photobioreactors) |  |  |  |  |  |
| Air Blower | 200 | m3·h-1 | € 3,191 | 0.99 |  |
|  | 1000 | m3·h-1 | € 5,959 | 3.96 |  |
|  | 2499 | m3·h-1 | € 11,788 | 11.15 |  |
| Degasser |  |  |  |  |  |
| Degasser | 60 | m3·h-1 | € 1,280 |  |  |
|  | 600 | m3·h-1 | € 2,641 |  |  |
| Harvesting |  |  |  |  |  |
| Centrifuge | 0.13 | m3·h-1 | € 28,463 | 1.1 |  |
|  | 2.1 | m3·h-1 | € 53,764 | 4 |  |
|  | 30 | m3·h-1 | € 180,000 | 18 |  |
|  | 70 | m3·h-1 | € 360,000 | 28 |  |
| Tilted Screen (120m2) | 200 | m3·h-1 | € 7,768 | 0.2 |  |
| Vacuum Suction filter belt | 2 | m3·h-1 | € 93,211 | 0.45 |  |
| Reactor Frames & Support |  |  |  |  |  |
| Steel Mesh Casing (Flat Panel) | 0.875 | kg·m-2 | € 650 ·ton-1 |  |  |
| Metal Poles (Vertical Tubular) | 3.8 | kg·m-1 | € 720 ·ton-1 |  |  |

### Appendix C: Biomass production cost breakdown

Table C.1 – Biomass production capacity, and breakdown of projected biomass production costs, in capital expenses (CAPEX) and operational expenses (OPEX) for 1, 10 and 100 ha production facility sizes located in Qatar (Al Khor).

|  |  | Biomass Capacity | Biomass Costs | CAPEX | OPEX | Initial Investment | Major Equipment | Construction | Raw Materials | Consumables | Energy | Labor | Wastewater Treatment | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | t･y-1 | €･kg-1 | €･kg-1 | €･kg-1 | M€ | €･kg-1 | €･kg-1 | €･kg-1 | €･kg-1 | €･kg-1 | €･kg-1 | €･kg-1 | €･kg-1 |
| Raceway pond | 1 ha | 51 | 14.7 | 2.1 | 12.6 | 1.3 | 0.5 | 1.6 | 0.4 | 0.1 | 0.1 | 6.9 | 0.8 | 4.3 |
|  | 10 ha | 509 | 4.9 | 1.4 | 3.5 | 8.2 | 0.3 | 1.0 | 0.4 | 0.1 | 0.1 | 1.2 | 0.8 | 1.0 |
|  | 100 ha | 5091 | 2.9 | 1.0 | 1.9 | 61.7 | 0.2 | 0.8 | 0.4 | 0.1 | 0.0 | 0.2 | 0.8 | 0.4 |
| Horizontal tubular | 1 ha | 40 | 21.4 | 5.1 | 16.2 | 2.6 | 0.9 | 4.3 | 0.8 | 0.5 | 0.2 | 8.8 | 0.2 | 5.7 |
|  | 10 ha | 398 | 7.5 | 3.0 | 4.5 | 15.4 | 0.5 | 2.5 | 0.8 | 0.5 | 0.2 | 1.5 | 0.2 | 1.4 |
|  | 100 ha | 3977 | 4.7 | 2.3 | 2.4 | 116.8 | 0.4 | 1.9 | 0.8 | 0.5 | 0.1 | 0.2 | 0.2 | 0.6 |
| Flat panel | 1 ha | 72 | 12.1 | 2.8 | 9.3 | 2.6 | 0.5 | 2.3 | 0.7 | 0.1 | 0.2 | 4.9 | 0.2 | 3.2 |
|  | 10 ha | 716 | 4.6 | 1.8 | 2.8 | 16.5 | 0.3 | 1.5 | 0.7 | 0.1 | 0.2 | 0.8 | 0.2 | 0.8 |
|  | 100 ha | 7159 | 3.1 | 1.5 | 1.6 | 136.4 | 0.3 | 1.2 | 0.7 | 0.1 | 0.1 | 0.1 | 0.2 | 0.4 |
| Vertical stacked tubular | 1 ha | 64 | 15.4 | 4.3 | 11.1 | 3.5 | 0.7 | 3.5 | 0.9 | 0.5 | 0.2 | 5.5 | 0.3 | 3.8 |
|  | 10 ha | 636 | 6.3 | 2.5 | 3.8 | 20.7 | 0.4 | 2.1 | 0.9 | 0.5 | 0.2 | 0.9 | 0.3 | 1.0 |
|  | 100 ha | 6629 | 4.3 | 2.0 | 2.3 | 168.9 | 0.3 | 1.6 | 0.9 | 0.4 | 0.1 | 0.1 | 0.2 | 0.5 |

### Appendix D: Biomass production cost reductions for different optimization strategies

Table D.1 – Biomass production cost reductions for the different optimization parameters compared to the base-case, assessed for each cultivation system at 100 ha scale in Qatar (Al Khor).

| Reactor System | Parameter | Reference Costs | Optimized Costs | Cost Reduction) |
| --- | --- | --- | --- | --- |
|  |  | €･kg-1 | €･kg-1 | % |
| Horizontal Tubular | Photosynthetic efficiency | 4.78 | 3.18 | -33.4% |
|  | Temperature Optima |  | 3.58 | -25.0% |
|  | Harvesting |  | 4.62 | -3.3% |
|  | Operational Days |  | 4.42 | -7.5% |
|  | Alternative Urea Source |  | 4.74 | -0.8% |
|  | Recovered CO2 |  | 4.44 | -7.0% |
|  | Wastewater treatment costs |  | 4.68 | -2.0% |
|  | Electricity costs |  | 4.71 | -1.4% |
| Vertical Stacked Tubular | Photosynthetic efficiency | 4.53 | 3.04 | -32.9% |
|  | Temperature Optima |  | 3.80 | -16.1% |
|  | Harvesting |  | 4.31 | -4.9% |
|  | Operational Days |  | 4.20 | -7.3% |
|  | Alternative Urea Source |  | 4.50 | -0.7% |
|  | Recovered CO2 |  | 4.20 | -7.3% |
|  | Wastewater treatment costs |  | 4.40 | -2.9% |
|  | Electricity costs |  | 4.47 | -1.4% |
| Flat Panel | Photosynthetic efficiency | 3.19 | 1.83 | -42.7% |
|  | Temperature Optima |  | 2.53 | -20.7% |
|  | Harvesting |  | 3.04 | -4.8% |
|  | Operational Days |  | 2.98 | -6.6% |
|  | Alternative Urea Source |  | 3.16 | -1.0% |
|  | Recovered CO2 |  | 2.86 | -10.4% |
|  | Wastewater treatment costs |  | 3.10 | -2.9% |
|  | Electricity costs |  | 3.13 | -1.9% |
| Raceway Pond | Photosynthetic efficiency | 2.95 | 1.70 | -42.3% |
|  | Temperature Optima |  | 2.95 | 0.0% |
|  | Harvesting |  | 2.45 | -16.9% |
|  | Operational Days |  | 2.80 | -5.0% |
|  | Alternative Urea Source |  | 2.91 | -1.3% |
|  | Recovered CO2 |  | 2.61 | -11.5% |
|  | Wastewater treatment costs |  | 2.54 | -13.8% |
|  | Electricity costs |  | 2.93 | -0.6% |